Manitoba Orienteering Association
SPORTident Equipment Usage

This document describes what is in the SPORTident kits and how to use the equipment. This document is meant for club type events where a computer is not required for doing the results. If you have any questions on the usage of the equipment please see the equipment manager.

Kit Contents
There are 3 clear bin sets of equipment and they each have the following:

	In Blue Cases

	ID
	SET 1 (100 series)
	SET 2 (200 series)
	SET 3 (31-70 series)

	Control Units
	Control #s 100 to 140
	Controls #s 200 to 237
	Controls #s 31 to 70

	Common items
	Clear, Check, Start and Finish, Time Master Unit (all blue controls)

	
	Coupling, Service OFF, Clear Backup, Print Results, Start (Purple) Sticks

	
	Splits printer with Charger and cables, Extra Rolls of paper for printer

	
	Printer Download Station (Black with yellow stripe) with attached cable.

	
	Download Station (red)

	15 Si Sticks
	247103, 247104, 247105, 247106, 247107, 247108,

247110, 247111, 247113, 247114, 247115, 247116,

247117, 247118, 247119
	247120, 247122, 247123, 247124, 247126, 247129,

247130, 247131, 247132, 247133, 247134, 247135,

247136, 247137, 247138
	

	In Clear Bins

	Compasses
	001-012 & 101-123
	013-024
	025-036

	40 Control Flags with Punches
	Ordinary Flags
	Reflective Flags
	Reflective Flags

	Additional Equipment to take with Sets

	45 Si Control stands (5 tubes with 9 per tube)
	Blue Aluminum Stakes, plus 4 stakes for Start, Finish, Clear and Check
	Yellow Fiberglass stakes
	Red Aluminum Stakes, plus 4 stakes for Start, Finish, Clear and Check

	Minor Events
	Power Pack (Energizer Arc5) and charger

	
	Road signs (3 wooden and 10 plastic available)

	
	Epsilon Printer

	
	MOA computer (and extra download and SI-cards)

	Major Events
	Bunting and hard container of fiberglass stakes

	
	10x10 MOA branded canopy tents (x3 and 3 sides)

	
	Start Banner

	
	Inkjet Printer (HP OfficeJet250 Mobile All-in-one) in Pelican Case

	
	Results Display and Start Monitor with Stands

	
	Podiums


If anything is missing from the kits or broken, please let the equipment manager know.
Storage

1. Please make sure the units and flags are dry before packing the tub.
2. Do not store the power pack or SI units outside when the temperature is below freezing.

3. When storing the control units, alternate the units so that the holes do not line up on 2 units adjacent to each other. See Figure 1.
[image: image1.jpg]


 [image: image2.jpg]







Figure 1



Figure 2
Before Event
1. Charge Power Pack. Plug charger into power pack and an outlet and let unit charge for at least 24 hours. It will not hurt the unit to leave it plugged in for longer. The green light on the front will blink when the unit is charging. See Figure 3 for how to store the charger.
2. Using the Time Master unit set the time on all the control station units that you are using. See separate section on how to do this. Note the Time Master unit is set with Daylight Savings Time (DST). If you are using the controls during standard time all relative times will still be correct but the actual times will be one hour later.

[image: image3.jpg]



Figure 3
During Event

1. Use the Time Master unit to set the time on the start and finish punches. See separate section on how to do this.

2. Use the purple clear backup stick to clear the printer download station. The unit will beep when the unit is cleared.

3. Setup the splits printer as follows (see Figure 4):

a. Connect the cable attached to the printer download station to the printer using the black or grey cable.
b. Connect the charger to the printer and plug the charger into the power pack.

c. Turn on the power pack.

d. Use the purple print results stick to verify the printer unit is cleared.

4. When competitors download have them download twice. The competitor keeps one set and the organizer can keep the second one to check that the correct control codes were punched.
5. See below if you need to change paper in the printer.

6. After the event DO NOT clear the printer unit until the results have been downloaded.

7. Ensure the power pack is turned off when finished.

Note for mass start events use the purple Start stick in the printer download station at the time of the mass start. This setting will mean that everyone does not need to punch the start station. In this case you should also time master the printer download station beforehand.

[image: image4.jpg]



Figure 4
Setting the Time in the Si Units

1. Insert the purple Service Off stick into the Time Master unit twice. The display will show TIMEMA. See Figure 5.
2. Insert the coupling stick into the Time Master unit as shown in Figure 6.
3. Couple the Time Master unit to the control unit, as shown in Figure 7, until you get 2 beeps. This usually takes only a few seconds. 
4. Use the Service off stick to turn off the control unit.

5. Repeat steps 3 and 4 for additional units.

6. When done insert the Service off stick into the Time Master unit until the unit turns off.


Note that you only need to set the time in the control units and the start and finish units. The clear, check and downloads stations do not need to be set.

[image: image5.jpg]


 [image: image6.jpg]


 [image: image7.jpg]



Figure 5



Figure 6


Figure 7
Changing the printer paper

1. Open the printer lid gently as shown in Figure 8. The tabs on the lid are very fragile so please be careful.

2. Remove any old paper and insert new roll with paper coming out as shown in Figure 9.

3. Gently push down on the front of the lid to close – see Figure 10.
[image: image8.jpg]


 [image: image9.jpg]


 [image: image10.jpg]



Figure 8



Figure 9


Figure 10

SPORTident Equipment Usage

3 January 2010
Page 3 of 3

